PROGRAM

org 0000h

mov a,#0ffh

mov p3,a

clr p3.0

clr p3.1

loop3:   setb p2.3

acall comm

acall del

mov a,#38h

acall comm

acall del

mov a,#0eh

acall comm

acall del

mov a,#01h

acall comm

acall del

mov a,#06h

acall comm

acall del

mov a,#80h

mov a,#'G'

acall dat

acall del

mov a,#'A'

acall dat

acall del

mov a,#'S'

acall dat

acall del

mov a,#' '

acall dat

acall del

mov a,#'D'

acall dat

acall del

mov a,#'E'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'E'

acall dat

acall del

mov a,#'C'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'I'

acall dat

acall del

mov a,#'O'

acall dat

acall del

mov a,#'N'

acall dat

acall del

mov a,#0c0h

acall comm

acall del

mov a,#'S'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'A'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'U'

acall dat

acall del

mov a,#'S'

acall dat

acall del

mov a,#':'

acall dat

acall del

mov a,#'A'

acall dat

acall del

mov a,#'C'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'I'

acall dat

acall del

mov a,#'V'

acall dat

acall del

mov a,#'E'

acall dat

acall del

clr p2.0

mov a,#00h

mov p1,a

loop:     jnb p3.0,loop

loop1:   clr p2.3

mov a,#01h

acall comm

acall del

mov a,#80h

acall comm

acall del

mov a,#'A'

acall dat

acall del

mov a,#'L'

acall dat

acall del

mov a,#'E'

acall dat

acall del

mov a,#'R'

acall dat

acall del

mov a,#'T'

acall dat

acall del

mov a,#'!'

acall dat

acall del

mov a,#' '

acall dat

acall del

mov a,#'G'

acall dat

acall del

mov a,#'A'

acall dat

acall del

mov a,#'S'

acall dat

acall del

mov a,#' '

acall dat

acall del

mov a,#'L'

acall dat

acall del

mov a,#'E'

acall dat

acall del

mov a,#'A'

acall dat

acall del

mov a,#'K'

acall dat

acall del

mov a,#'!'

acall dat

acall del

mov a,#'!'

acall dat

acall del

mov a,#01h

acall comm

acall del

jnb p3.1,rep

ljmp loop3

rep:        ljmp loop1

comm:   mov p1,a

clr p2.0

clr p2.1

setb p2.2

acall del

clr p2.2

ret

dat:       mov p1,a

setb p2.0

clr p2.1

setb p2.2

acall del

clr p2.2

ret

del:        mov r3,#50

h1:mov r4,#255

here:djnz r4,here

djnz r3,h1

ret

end

LCD COMMANDS

| Hex Code | Command to LCD Instruction Register |
| --- | --- |
| 01 | Clear screen display |
| 02 | Return home |
| 04 | Decrement cursor |
| 06 | Increment cursor |
| 0E | Display ON, Cursor ON |
| 80 | Force the cursor to the beginning of the 1st line |
| C0 | Force cursor to the beginning of the 2nd line |
| 38 | Use 2 lines and 5x7 matrix |